초임계 이산화탄소를 이용한 Ephedra sinica로부터 Ephedrine

유도체 추출에 미치는 공용매 효과

유정환, 노민정, 유기풍, 진영원*, 최영해*, 김진웅*

서강대학교 공과대학 화학공학과

서울대학교 약학대학 약학과*

Effect of Modifiers on the Supercritical CO2 Extraction of Ephedrine Derivatives from Ephedra sinica

Jung Hwan Ryu, Min Jeong Noh, Ki-Pung Yoo,

Young-Won Chin, * Young Hae Choi* and Jinwoong Kim*

Department of Chemical Engineering, Sogang University

College of Pharmacy, Seoul National University

Introduction

In oriental traditional medicine Ephedra Herba has long been used for diaphoretic, anti-asthmatic, and diuretic as well as for the treatment of bronchitis and acute nephritic edema. Especially the alkaloids from Ephedra species (i.e., ephedra alkaloids) are used in modern medicine for asthma, influenza and inflammation. The main active alkaloids are consist of L-ephedrine, D-pseudoephedrine, D-norpseudo-ephedrine, L-norephedrine, methylephedrines, L-methylephedrine and D-methyl pseudoephedrine. Thus the quality of the crude ephedrine-based drug is frequently evaluated in terms of its alkaloid content. [1,2,3,4]

To date, the selective isolation of such a great variety of ephedrine derivatives from Ephedra Herba has been solely based on the conventional liquid solvent extraction. However, when one adopts such a conventional technique for the separation of such alkaloids, the use of multistage extraction steps is usually necessary [1,2,3,4]. Thus toward searching for an alternative extraction process, the CO2-based supercritical fluid extraction (SFE) method was tested for the selective extraction of  the target ephedrine derivatives (i.e., ephedrine, pseudoephedrine, and methylephedrine) from Ephedra sinica Stapf.

Since in recent years, much attention has been placed on feasibility studies of replacing conventional liquid solvent extractionby SFE for obtaining phytochemicals from natural resources[5].

In fact, upon the present authors’ preliminary experiment, polar substances such as ephedrines were not soluble in pure supercritical CO2. Thus, in order to improve those limitation of  pure CO2 as a supercritical solvent, illustratively the effect of addition of least amount of polarity modifiers to CO2 was experimentally investigated.

EXPERIMENTAL

Plant material. Aerial part of Ephedra sinica Stapf were obtained from Korea Export and Import Federation of Drugs. The sample was dried for 24 hr at 40 oC in an oven and powdered.

Standards. Reagent-grade L-ephedrine hydrochloride, D-pseudoephedrine hydrochloride, and L-methylephedrine hydrochloride were purchases from Sigma Chem. Co. (St. Louis, MO, USA).

CO2 -based supercritical solvent extraction. Extraction was performed by the ISCO supercritical fluid extractor model SFX-3560 (Lincoln, NE, USA). For each run, 100 mg sample of Ephedra Herba was loaded into 10 mL extraction cell. Experimental temperature was in the ranges from 40 to 80 oC and pressure was in the ranges from 2000 to 5000 psi. The restrictor in the apparatus was kept at 140 oC.

Each 100 l modifier was added to the extraction cell prior to SFE. In experiment, the static time was 10 min and flow rate of CO2 was 1.0 mL/min, and the extraction was carried out for 30 min. After each extraction run, the extracted substances attached through the line was rinsed with pure CO2 for 20 min. The extract was collected by the methanol cold trap. The combined extract was evaporated and then diluted with 1 mL methanol. The 40 mL orcinol (1 mg/mL, internal standard) was added and evaporated for later gas chromatographic analysis. To assure the reliability the measured data, experiments were conducted in triplicate for each condition.

Liquid solvent extraction. 1 g Ephedra sinica was extracted by using 70 mL of 0.5% H2SO4 for 12 hrs at room temperature. It was then filtered . 6 M NaOH was added and the pH of the mixture was adjusted within the ranges from 11 to 13, then 16 g of sodium chloride was added. The mixture was extracted with 100 mL(40, 20, 20, 20 mL) of ethyl ether. After evaporation of ethyl ether, the extract was dissolved by 10 mL methanol and then diluted by 10 times with methanol[3]. And 40 mL orcinol was added and evaporated. The experiments were conducted in triplicate.

Gas chromatographic analysis. A HP 5890 II gas chromatography (Hewlett Packard, Avondale, PA, USA) equipped with a flammable ionization detector (FID) and HP 3395 integrator was employed. An Ultra-1 (length 25 m, i.d. 0.32 mm, film thickness 0.25 mm) was used with detector temperature at 280 oC, injector temperature 220 oC, and oven temperature initially at 90 oC. The oven temperature was held for 1 min and heated by 3 oC/min to 124 oC and held for 3 min and then to heated to 280 oC at a rate of 20 oC/min [6]. The carrier gas was helium at a flow rate of 2.4 mL/min. The split ratio was 1:50. MSTFA (100 L, Sigma) was added to the extract under a slow stream of nitrogen. The mixture was reacted at 75 oC for 2 hr and 1 mL of the solution was injected. The yield of each extraction for ephedrine, pseudoephedrine, and methylephedrine was calculated by each standard calibration curve.

RESULTS AND DISCUSSION

An illustrative effect of modifiers in the CO2-based supercritical extraction was shown in Fig. 1. While some modifiers such as ethyl acetate, acetone and chloroform showed almost no significant enhanced effect in the extraction of ephedrine, pseudoephedrine, and methylephedrine, such modifiers as diethylamine, methanol and ehanol showed noticeable effect of the polarity change and thus increased extraction of ephedrine derivatives. In particular, addition of diethyl- amine showed the most enhanced modifier effect than others. In addition, when we used diethylamine as modifier, ephedrine was more selectively extracted than the case of psuedoephedrine with comparison with the other modifiers. The cause might be that methanol and ethanol simply tend to increase the polarity of supercritical CO2, while diethylamine seemed to neutralize the charged ephedrine derivatives (in general alkaloids exist as a salt form such as ephedrine sulfate in plant tissues)[7] by ion-pair formation.

Based on above screening tests to search a proper modifier, we found that diethylamine is one of good modifers for CO2. Thus, by choosing diethylamine, further extraction was carried out by varying the volume percentage of addition of diethylamine from 0.5 to 4% in supercritical CO2. The effect of increased percentage of diethylamine in CO2 extraction of ephedrine was shown accordingly in Fig. 2. As one can see from this figure, the addition of least amount of diethylamine (50 L) found to be appropriate for the enhanced and selective extraction of ephedrine from Ephedra sinica.

In investigating further the effect of a fixed amount of diethylamine addition (0.5 volume %) to CO2, experiment was carried out under various temperatures and pressures for the SFE of ephedrine, pseudoephedrine, and methylephedrine from Ephedra sinica. Extraction yields of ephedrine, pseudoephedrine, and methylephedrine were increased with increasing temperature at constant pressure, while they had maxima at 4000 psi of all temperature conditions.

Finally, comparative efficiencies of extractions of ephedrine derivatives by both the CO2-based SFE and by the conventional liquid solvent extraction were evaluated. Upon comparison, the SFE were not especially good when compaed in terms of general extraction yield (approximately 10-30 % of the case of liquid solvent extraction). However, the SFE was proven to be simple, fast and furthermore, when one used a modifier (i.e., diethylamine), it was possible to extract ephedrine more selectively than psedoepherine from Ephedra sinica (Fig. 3).

REFERENCES

1.Yamasaki, K., Fujita, K., Sakamiti, M., Okada, K., Yoshida M.and Tanaka, O. Chem. Pharm. Bull. 22, 2889-2902.(1974)

2.Moriyasu, M., Endo, M., Kanazawa, R., Hashimato, Y., Kato, A. and Mizuno, M. Chem. Pharm. Bull. 32, 744-747.(1984)

3.Zhang, J., Tian, Z. and Lou. Z. Planta Med. 54, 69-70. (1988)

4.Cui, J., Zhou, T., Zhang, J. and Lou, Z. Phytochem. Anal. 2, 116-119.(1991)

5.Bevan, C. D. and Marshall, P. S. Nat. Prod. Rep. 11, 451-466. (1994)

6.Modey, W. K., Mulholand, D. A. and Raynor, M. W. Phytochem. Anal. 7, 1-15. (1996)

7. Tyler, V. E., Brady, L. R., Robbers, J. E. Pharmacognosy, 9th ed. pp. 240-242, Lea & Febiger, Philadelphia.(1988)

Fig. 1. Gas chromatogram of an extract of Ephedra sinica with carbondioxide at 80 oC and 5000psi. (A) pure carbondioxide, (B) modified with ethanol, (C) modified with methanol, (D) modified with diethylamine.

ME: Methylephedrine, PE : Pseudoephedrine, E : Ephedrine, IS : Internal standard

Fig. 2. Amounts of ephedrine in        Fig. 3. Gas chromatogram of an extract

mg/g extracted from E.sinica by        of Ephedra sinica utilizing liquid solvent

the SFCO2 modified with diethyl-       (10 times diluted) (A) and carbondioxide

amine.                              modified with diethylamine at 80oC and

5000 psi (B).