클러스터링 기법을 이용한 많은 운전 조건을 갖는 공정의

모니터링 방법

황대희, 한종훈

포항공과대학교 화학공학과, 자동화 연구 센터

Process Monitoring Methodology of the Processes with multi operating conditions using clustering analysis

Dae-Hee Hwang, Chonghun Han

Automation Research Center and Dept. of Chemical Eng. POSTECH

Instruction

Due to an increasing interest of safety and consistent product quality, it has been an essential precondition to monitor process performance in order to survive and grow in the competing global market. MSPC has been dedicated to this goal for the past years, replacing the traditional chartings, so that we can overcome the difficulties from strong correlation in the multivariate case. They are commonly used to monitor if process variables are kept at a steady state mode. In practical, however, almost continuous processes usually go through the continuous changes of operation condition by set-point changes of controlled variables or operators’ actions to maintain process stability. The state variables in the process never stay at one steady state level, but they move along the changing levels. Therefore, the traditional application of Principal component Analysis (PCA) can not be used. Some other strategy will be necessary for its proper application. Gallagher(1997) proposed an online adaptive method for the semiconductor etch process, a nonstationary time varying process. Gorgeokis(1996) developed a dynamic PCA model applicable to any nonstationary time varying process. However, they are not applicable to practical processes involving a lot of operation changes for the various reasons. This paper proposes an alternate method designed to develop an offline PCA model to monitor a nonstationary process. We keep the past information as a clustered form in a super PCA model unlike an adaptive model developed by Gallagher. The methodology involves clustering algorithm to represent a steady state mode as a cluster. Although a lot of clustering algorithms are available, for an efficient clustering scheme, we used a hierarchical clustering algorithm based on Self-Organizing Maps (SOM), an interesting topic in the neural network field, involving the variance tuning at each cluster.

Theory

Step 1.  Hierarchical clustering.    We selected the normal operation data from original operation data using our proposing selection algorithm[4]. The selected data, (x1, x2, ..., xk), came from a lot of operation conditions would not show a normal distribution in k dimension space. Rather, a lot of small clusters are assembled in some region and scattered in other region, so that shape a total outlook in k dimensional data space, just as a grape does.

We cluster the selected normal operation data into n operation modes, n clusters. We assume that at each mode a steady state assumption should be reasonable. The clustering has been explored by hierarchical clustering algorithm[3]. Unlike conventional clustering methods, this method produces a family of hierarchical cluster decomposition. The original data cluster decomposes into n1 cluster at the first level and the decomposition process can be iterated, with each cluster successively being decomposed into n2, n3, ... in turn, so that one cluster is broken down into many lower sized clusters. This is called a cluster decomposition tree. Fig 1 shows how hierarchical decomposition of a original cluster can be done.

We follow the approach of growing a large tree with pure terminal nodes and selectively pruning it back or combine some terminal nodes. This method divides the original data sample space into n terminal clusters in a hierarchical structure.

Once a large tree has been grown, the outliers have been eliminated so that they can not prevent the means and covariance matrixes from being biased or modifying the true shape of cluster. Then, we prune some branch or cut some nodes and combine them as one node to obtain the best tree structure. We combine the nodes locating so nearby to each other that there can be a large probability of error (eqn (1)) associated with the classification scheme calculated by mahalanobis distance in eqn (2).

p(eij)=12πexp(−y22)dy                                          (1)

rij=(mi−mj)'C−1(mi−mj)                                               (2)

where mi and mj are the means in the ith and ith cluster, respectively, and C is a covariance matrix. We combine the clusters into one cluster if their probability of error in the classification performance is larger than 10%, i.e., if the mahalanobis distance(rij) is less than about 5.

p(eij)≥0.1orrij≤5                                                     (3)

We prune a branch that has small cluster as a terminal node and we can ignore p(wi), comparing the other terminal nodes, as eqn (4) shows.

p(w)<<p(wj)            for    j=1, 2, ...., n, and i≠j                         (4)

Step 2.    PCA modeling    The first step in the super PCA modeling is to translate each cluster into the origin and normalize the translated cluster by dividing its data matrix by its norm as in eqn (5). The norm of matrix is defined as the largest singular value.

Xtni=(Xi−1miT)si=Xisi−1si1miT                                             (5)

where Xtniis a translated and normalized data matrix, mi and si are a mean vector and the largest singular value in the ith cluster. The next step in the super PCA modeling is to put all translated and normalized data matrixes together to make a super data matrix. Eqn (6) explains this process.

Xsup=Xtn1TXtn2T...XtnnTT                                           (6)

where Xtni is the ith translated and normalized data matrix and Xsup is a super data matrix.

XsupTXsup=X1TX2T...XnTX1TX2T...XnTT=X1TX1+X2TX2+...+XnTXn=VaΛaVaT                    (7)

Step 3.  Process monitoring    When monitoring the process preformance with a new operation data using a super PCA model, we assign the new observation to one of clusters using clustering model developed by BP. Once it was assigned to a certain cluster, it goes through the same linear transformation, i.e., translation and normalization.

xnewn=(xnew−1miT)si=xnewsi−1si1miT                                             (8)

Then, we can calculate T2and SPE values by projecting it into the PCs of the super data matrix and obtaining xnewnin eqn (8) and (9).

T2=xnewnVaTxnewnVa          SPE=xnewn−xnewn2                     (9)

Case study for blast furnace    A Blast Furnace(BF) is a large reactor producing pig iron. A lot of reactions occur in complicated ways, simultaneously with physical phase transitions, heat, and mass transfer inside the BF. Thus, the process has highly nonlinear characteristics and strong correlation among the variables. For those reason, a lot of monitoring system based on mathematical modeling have been failed reasonable answer enough to satisfy process operators.

We have collected the last six month operation data from the fourth blast furnace of POSCO, an iron making company, at pohang in Korea. The measured variables are composed of 329 temperatures through the whole reactor, 31 pressures in shaft region, the upper part of the reactor, and 4 gas concentrations at the top. We selected 2506 normal operation data out of 4193 original observations by the proposed selection algorithm.

Step 1.  Hierarchical clustering    The selected normal operation data matrix has been clustered by hierarchical clustering algorithm. The best clustering tree has been produced, where we had pruned some branches and combined the terminal nodes in terms of the mahalanobis distance. We developed a clustering model using BP. The number of nodes in the input layer is 377. And the node number of hidden layers has been set 120 for the first hidden layer and 80 for the second hidden layer.

Step 2.    PCA modeling  We developed a super PCA model by following step 3.

Step 3.    Process monitoring    The first test set is made from the raw data for the second cluster. As we can see in Fig 15, the abnormal operations eliminated by the selection algorithm came up to the control limits in SPE control plot. For the second test set, we put some observation from the first cluster together with the data from one of the pruned clusters, C211, which has been pruned purposely to apply our updating algorithm and check if it works properly. As we can see in Fig 16, the observation from the pruned cluster has been detected by continuously crossing out the control limits in the end of T2 and SPE control plot, though it is not the cluster composed of abnormal operations. Seeing the observations continuously crossing out the limit in SPE plot, it can be concluded that set point or actions has been taken and a new operation condition has been established.

Conclusion    The success of monitoring strategy strongly depends on the availability of long term data and their quality. In the case of multilevel operation, we presented the methods to check them using the distribution of clusters in the past operation range and signal to noise ratio defined as the validity of variables.

The development of PCA monitoring scheme for multilevel operation has been possible by clustering, translation and normalization of the clusters, and developing a super PCA model. It has been sensitive to any abnormality in all operation conditions included in the super PCA model. In other words, it was sensitive to the observations violating correlation structure within a cluster and even to the observations of the sparse region in the selected data space, which has not been possible, when we apply PCA for the whole selected data. Furthermore, the initial super PCA model has detected a new operation condition and can be updated to include the new normal operation condition by the proposed update algorithm. Then, it was confirmed that it really works.

Specially, the sustained abnormalities or the changes of operation condition make it impossible to diagnose the faults using traditional T2 and SPE contribution plot. It was overcome using the mapping technique.

Reference

- Ljung, L., “System Identification theory for the users,” prentice hall, (1987)

- Jackson J.E., “A User’s Guide To Principal Components,” John Wiley & Sons,             (1991)

- R.A Johnson and D.W. Wichen, “Applied Multivariate Statistical Analysis,” prentice hall, (1987)

- A. Santen, G.L.M. Koot and L.C. Zullo, “Statistical Data Analysis of a Chemical

Plant,” PSE’97-ESCAPE-7, Suppl., 21, pp. S1123-1128, (1997)

Mother

cluster

C0

....

Fig1 Hierarchical clustering               Fig2 Hierarchical clustering for blast furnace

Fig3 Control charts for the first(left) and second(right) test set