목재분진의 독성물질이 환경홀몬특성에 미치는 영향 ( I )

설 수 덕*.이 내 우.이 병 선. Dino L. Pisaniello**

* 동아대학교 화학공학과.

부경대학교 안전공학과.

** 남호주 Adelaide대학교 공중보건학과

Effects of Environmental Hormone Characteristics by Toxic Substances in Wooddust ( I )

Soo-duk Seul*.Nae-woo Lee.Byung-sun Lee.Dino L. Pisaniello**

Department of Chamical Engineering, Dong-a University, Pusan, Korea*

Department of Safety Engineering, Pukyong National University, Pusan, 608-739, Korea

** Department of Public Health, University of Adelaide, SA 5005

- 서  론

Tannins are complex polyphenolic substances present in plants. They may be classified as either hydrolysable or condensed tannins, and extracts of the latter type have displayed carcinogenic properties in animal studies (Hausen, 1981). The International Agency for Research on Cancer has determined that exposure to airborne wood dust, especially hardwood dust during furniture manufacture, is a cause of sino-nasal cancer, notably nasal adenocarcinoma. It appears that the carcinogenicity potency is in the wood itself and not in decomposition products or in the substances applied to the wood. Tannins have been implicated because hardwoods are thought to contain proportionally more tannin than softwoods. Furthermore, it has been suggested that the elevated sino-nasal cancer risk in shoemakers may also be due to tannin exposure (Bianco and Savolainen, 1994).

In a recent Australian nasal cytological study, it was found that hardwood exposure was more likely to be associated with nasal metaplasia in experienced woodworkers (Pisaniello et al, 1995).

Thus it is of interest to determine tannin exposures in woodworking processes, and to assess whether tannin exposure, as distinct from wood dust, is a better indicator of sino-nasal risk.

This paper examines the tannin content of a range of woods used in Australian furniture manufacture, and also reports airborne tannin levels in two factories.

2. 실험 방법

Inhalable wood dust was measured according to AS 3640-1989, utilising a sampling strategy reported by Pisaniello et al (1991). Tannin values were determined in triplicate according to methods described in Bianco and Savolainen (1994) and Lau et al (1989).

Wood dust samples were extracted with 50% methanol for one hour. All solutions were filtered prior to spectrophotometric analysis, and all reagents were of analytical-reagent grade. There was no significant influence of extraction time (30 min – 6hrs), or air sampling filter type (PTFE, cellulose ester, glass fibre or PVC) on measured tannin concentration.

A nasal examination (anterior rhinoscopy) was performed at the workplace, noting any mucosal or anatomical abnormalities. Brush cytological specimens were then obtained from the anterior region of the middle turbinate of the nasal cavity judged to have the best air flow. The specimens were immediately applied to glass slides and fixed in 96% ethanol without air drying.

- 결과 및 고찰

- The tannin concentration in wood samples varied from 0.43 to 8.72 mg tannic acid equivalent per gram of wood, for reconstituted softwood and turpentine (Syncarpia glomuliferia) respectively (Table I). This range is somewhat lower than that reported by Bianco and Savolainen, (1994), i.e. 1.6 – 80 mg tannic acid equivalent g-1 of wood, although the wood species are not strictly comparable.

- Table II indicates that hardwoods generally contain more tannin than softwoods, which is consistent with previous research.

- Measured concentrations of airborne wood dust and tannin are given in Table III. Note that the inhalabale dust concentrations are similar to those published elsewhere, again demonstrating excessive exposures from sanding operations (Pisaniello et al, 1991). Airborne tannin was somewhat lower than that reported by Bianco and Savolainen (1994), and this result may be attributable to the different woods encountered. In the latter study, oak displayed a disproportionately high tannin content. On the other hand, our investigations showed that careful filtration of solutions is necessary to avoid suspended particulate giving high absorbances.

- This work is in progress, of course, the relationship between tannin concentration and nasal cytological score is examined.

- Nasal examination of woodworkers and controls did not reveal any significant clinical abnormalities in either group. Cuboidal metaplasia was present in 23 furniture workers(46%), compared with 10 controls(20%). This prevalence rate ratio of 2.3 was statistically significant.

- Among woodworkers, cuboidal metaplasia was more common for those historically exposed to hardwoods and to dust levels greater than 2 mg/m3. All subjects with cuboidals metaplasia were also found to have squamous metaplasia.

- According to nasal cytology score table, the highest score was 4, corresponding to atypical squamous metaplasia. The prevalence of squamous metaplasia was slightly greater in the furniture workers than in the controls.The mean scores for the woodworkers and controls were 2.1 and 1.7 respectively.

- So many odds ratios, for examples, between wood exposure , historical tannin levels, current tannin levels, woodwork experiences and cytology score were investigated. The result of odds ratio between years of woodwork experience and cytology score was 1.83. This value was statistically significant. This reveals that significant clinical abnormalities mainly will be influenced from the years of woodwork experiences.

- 결론

This study has yielded new information on tannin exposures in the woodworking industry. Also the result of odds ratio between years of woodwork experience and cytology score was obtained as1.83. The value was statistically significant. This reveals that significant clinical abnormalities mainly will be influenced from the years of woodwork experiences.

Further research is required to evaluate the significance of the data, for the purposes of evaluating risk and standard setting.

참고 문헌

1) Bianco MA, Savolainen H, 1994, Woodworkers’ exposure to tannins, J. Appl Toxicol, 14: 293-295.

2) Hausen BM, 1981, Woods injurious to human health: A  manual, Water de Gruyter  & Co.  Berlin.

3) Lau OW, Luk S-F, Huang HL, 1989, Spectrophotometric determination of tannins in tea and beer  samples  with iron(III) and 1,10 phenanthroline as reagents, Analyst  114: 631-3.

4) Pisaniello DL, Connell KE, Muriale L, 1991, Wood dust exposure during furniture   manufacture – Results

from an Australian survey and considerations for TLV development. Am. Ind. Hyg. Assoc. J 52: 485-92

5) Pisaniello DL, Gun RT, Tkaczuk MN, Schultz MR and Stevens M, 1995, Nasal cytology in Australian

furniture woodworkers, Aust. J. Otolaryngol., 2: 137-141.

Table I: Tannin content of wood samples (mg tannic acid equivalent  g-1 of  wood)

| Classification* | Name |  | Tannin conc. (mg /g dust ) | Origin |
| --- | --- | --- | --- | --- |
|  | Common trade | ###### Botanical |  |  |
| Soft wood | Cedar, western red Fir, Douglas Pine, celery-top Pine, hoop Pine, Huon Pine, kauri Pine, lodgepole Pine, radiate | Thuja plicata Pseudotsuga menziesii Phyllocaldus asplenifolius Araucaria cunninghamii Dacrydium franklinii Agathis palmerstoni Pinus contorta Pinus radiata | 1.76–4.08 1.42 2.74 0.65 0.85 0.66 0.61 1.24 | 2)3) 1) 1) 1)14) 1) 1) 1) 2) 1) 3) 4) 5) |

| Hard wood | Aningeria Ash, alpine Ash, American Ash, mountain Ash, European Ash, silver Ash, white Bean, black Beech, European Beech, myrtle Blackbutt. W.A. Black wood Cherry Geronggang Gum, blue, Sydney Gum, red, river Imbuia Iroko Jarrah Karri Mahogany, American Mahogany, Brazil Mahogany, Fijian Maple, QLD Maple, sugar Meranti, dark, red Meranti, red Meranti, yellow Nyatoh Oak, American red Oak, American white Obeche Purpleheart Ramin Rosewood Sallywood, brown Sheoak, W.A. Tasmanian oak Teak Turpentine Walnut, American Walnut, European | Sapotaceae Eucalyptus delegatensis Fraxinus americana Eucalyptus regnans Fraxinus excelsior Flindersia bourjotiana Eucalyptus fraxinoides Castanospermum australe Fagus sylvatica Nothofagus cunninghamii Eucalyptus patens Acacia melanoxylon Prunus avium Cratoxylon arborescens Eucalyptus saligna Eucalyptus camaldulensis Phoebe porosa Chlorophora excelsa Eucalyptus marginata Eucalyptus diversicolor Swietenia macrophylla Carapa guianensis Sweitenia macrophylla Flindersia brayleyana Acer saccharum Shorea pauciflora Shorea Shorea faguetiana Sapotaceae Quercus Quercus Triplochiton scleroxylon Peltogyne venosa Gonystylus macrophyllus Pterocarpus indicus Acacia aulacocarpa Casuarina fraserana Eucalyptus delegatensis , Eucalyptus regnans Tectona grandis Syncarpia glomuliferia Endiandra palmerstonii Juglans nigra | 1.08 1.10 0.63 1.21-1.54 1.37 0.48 0.67 1.94-1.99 0.27-1.17 1.13 4.73 5.08-7.43 1.54 0.86 1.09 4.45-4.75 3.04 3.63 2.01 2.84 2.42 2.78 5.02 0.98 0.61-0.84 0.95 1.29 1.83 1.85 2.77 2.01-4.01 0.62 1.96 0.52 1.76 1.95 3.08 2.55 0.91 8.72 3.28 1.38 | 5) 1) 2)3) 1) 12)13) 1) 1) 1) 12) 1) 1) 1) 3)12)13) 7)8)9)10) 1) 1) 15) 5) 1) 1) 3) 15) 16) 1) 2) 3) 7)8)9)10) 7)8)9)10)13) 7)9) 7)8)9)10)11) 2)3) 2)3) 5) 3)5) 8)13) 7)8)10)13) 1) 1) 1) 1) 1) 1) 12)13) |
| --- | --- | --- | --- | --- |
| Reconstituted soft wood | Medium density F.B . HMR Particle board Chip board |  | 0.59 0.43 0.43 |  |

Notes for Table I:

1) Australia; 2) Canada; 3) USA;  4) New Zealand;  5) Africa;  6) Uganda;  7) Indonesia; 8) Malaysia;  9) Philippines;  10) Sabah;  11) Burma;  12) Europe;  13)  Asia;  14) New Guinea; 15) Brazil;

- Classification  and  name were cited from Australian Standard  2543 and 1148

Table II:  Tannin content by wood type

|  | Tannin concentration (mg /g dust) |  |  |
| --- | --- | --- | --- |
|  | Softwood | Hardwood | Recon.wood |
| Minimum | 0.61 | 0.27 | 0.43 |
| Maximum | 4.08 | 8.72 | 0.59 |
| Average | 1.68 | 2.22 | 0.48 |
| Median | 1.33 | 1.83 | 0.43 |

Table III:  Woodworker exposure to airborne wood  dusts  and  tannins

| Activity | Sampling Method | Principal Wood | Inhal.dust (mg/m3 ) | Tannin conc. (g / m3 ) | Remark |
| --- | --- | --- | --- | --- | --- |
| Sawing | Personal | Nyatoh, Jarrah, Tasmanian oak, Laminated Chipboard, Veneer, Plywood | 3.2 | 9.1 | circular saws, band saws, profilers, dimension saws, straight line edgers, trimmers. |
|  | Area |  | 0.72 | 3.9 |  |
| Cutting | Personal | Aningeria, MDF Laminated Chipboard Plywood, Nyatoh, Tasmanian oak | 2.4 | 4.6 | planers, thicknessers, moulders, shapers, mortisers, tenoners, spindle, moulders, copy lathes, router. |
|  | Area |  | 0.87 | 3.1 |  |
| Sanding | Personal | Laminated chip –board, Veneer Blackwood, Aningeria, Plywood, Ameri. Walnut, | 5.3 | 14.5 | Paper and block, Portable hand sand machines all kinds of sanding machines |
|  | Area |  | 2.2 | 5.0 |  |