# 스티렌 모노머 반응기의 수학적 모델링과 모사

임희진, 주은정, 이정석*, 장민호, 박선원

한국과학기술원, 화학공학과

*LG화학 SR연구소

# Mathematical Modeling and Simulation of Styrene Monomer Reactor

Heejin Lim, Eunjung Joo, Jeongseok Lee*, Minho Chang, Sunwon Park

Dept. of Chem. Eng., KAIST

*SR Research Center, LG Chem.

## Introduction

Most of the industrial styrene plants carry out the adiabatic dehydrogenation reaction in the multiple reactors in series. Even though these adiabatic reactors need more expensive equipment cost and operating cost, they are used in many companies because of their higher conversion of ethylbenzene, and the smaller usage of catalyst pellets.

Figure 1. Current adiabatic radial flow reactor

The rigorous styrene monomer(SM) reactor model is highly valuable because it can be used in optimizing the current operation, which has high cost due to the large amount of expensive high-pressure steam. But, only a few researchers have studied about industrial SM reactors because they have difficult geometry and unknown reaction mechanism[1].

Most previous researchers had studied about the development of new dehydrogenation catalyst and had experimented with plug-flow reactor using the known catalysts. Reaction models were formulated by Clough and Ramirez[2], Scheel and Crowep[3] and Hirano[4]. Crowe and Abdalla[5] studied about the characteristics of SM plug-flow reactor. Only Savoretti[6] proposed the non-adiabatic radial-flow reactor model.

In this study, reaction mechanism was used from Crowe et al. and the kinetic parameters were optimized using the real plant data.

## System Description

(1) Reaction mechanism

In the SM reactor, three major competing reactions are known: reactions (1), (2), and (3). Styrene is produced by dehydrogenation of ethylbenzene. These reactions are endothermic except (3).

C6H5CH2CH3↔C6H5CHCH2+H2		(1)

C6H5CH2CH3→C6H6+C2H4		(2)

C6H5CH2CH3+H2→C6H5CH3+CH4		(3)

In addition to these, there are three side reactions by thermal cracking at higher temperature.

H2O+12C2H4→CO+2H2			(4)

H2O+CH4→CO+3H2			(5)

H2O+CO→CO2+H2			(6)

Since the dehydrogenation of ethylbenzene is a reversible endothermic reaction, high styrene yield is favored by high temperature.

(2) Characteristics of reactor

The conversion of ethylbenzene and the selectivity of ethylbenzene to styrene are affected by reactor operating pressure, temperature, molar steam-to-hydrocarbon ratio (S/O ratio), and reactor LHSV (Liquid Hourly Space Velocity = [total hydrocarbon flow rate] / [total mass of catalyst]).

The desired reaction can be enhanced at the low operating pressure because the main reaction of styrene monomer production causes increment of the number of moles. The hydrocarbon feed to the reactor is the mixture of fresh ethylbenzene and recycled unconverted ethylbenzene. Prior to entering the reactor, this hydrocarbon feed is mixed with superheated steam in the adiabatic radial flow reactor. This steam acts not only as a heating medium but also a diluent. High S/O ratio and low operating pressure reduce the styrene partial pressure and increase the molar conversion of ethylbenzene. Molar conversion of ethylbenzene is also a function of temperature; higher temperature yields higher conversions in nearly a linear function. Therefore, an increment in conversion can be obtained by increasing the reactor temperature. Since the reaction of ethylbenzene to styrene is endothermic, it is carried out in multiple adiabatic radial bed reactors filled with catalysts. As for LHSV, molar conversion of ethylbenzene is an inverse function of this parameter since higher velocity means lower residence time. This in turn reduces molar conversion.

This SM plant has two serial ethylbenzene dehydrogenation reactors as shown in Figure 1. Between two reactors, there is a steam heat exchanger. In this heat exchanger, the product stream from the first reactor is reheated up to 630℃.

Table 1. Reaction model

Reaction amounts in reaction j,fj

f1=k1(PEB−PSTPH2KP) (7) f2=k2PEB (8) f3=k3PEBPH2 (9) f4=k4PH2OPET (10) f5=k5PH2OPME (11) f6=k6PT3PH2OPCO (12)

Reaction amounts of component I,fi,

fi=j=16νijFj (13)

Reaction rate constant

ki=k0exp−EiRT (14)

## Modeling

(1) Reaction model and Governing equations

The kinetic model for the reactions is shown in Table 1.

The adiabatic radial flow reactor is simplified using the following five assumptions.

- Quasi-steady state operation

- Ideal gas mixture: This assumption is valid because the styrene monomer reactor is operated at high temperature and low-pressure condition.

- Uniform distribution: The parabolic deflector inside of the plant makes the uniform flow pattern.

- No pressure drop and mass or heat diffusion in the axial direction: Only the pressure drop exists inside of catalyst bed in the radial direction.

- No reaction except catalyst bed

Mass balance

According to the previous assumptions, the shell mass balance in cylindrical coordinates is

giw2πrz|r−giw2πrz|r+Δr+j=1mfijπrΔrzMWi=0	(15)

The differential mass balance for species i is

−1r∂∂rrWgi+j=1mfijMWi=0, i=1NCgi=1		(16)

(a) Parameter optimization for first reactor

(b) Parameter optimization for second reactor

(c) Parameter test for first reactor

(d) Parameter test for second reactor

Figure 2. Parameter optimization and test

◆EB real □ EB calc △SM real ×SM calc ※ T real ◇ T calc

Temperature (oC)

Temperature (oC)

Temperature (oC)

Temperature (oC)

Because the mass flow rate F is constant (=2πrΔr*W) by assumption ③, (16) can be changed to the final mass balance, (17).

dgidr=1F2πrLj=1mfijMWi, i=1NCgi=1		(17)

Energy balance

To develop the thermal energy balance for the gas phase in the reactor the shell balance in the same coordinate to the mass balance is

q2πrz|r−q2πrz|r+Δr+2πrzpV|r−2πrzpV|r+Δr=0	(18)

Dividing by 2πrΔrz with Δr→0, equation (18) makes the following equation

Cpr∂∂rrWT=j=1m−ΔHjfj			(19)

Then equatin (19) can be rearranged with the similar way as mass balance like

dTdr=2πrLj=1m−ΔHjfjFCp			(20)

Chemkin subroutine is used to calculate the heat capacities, Cp.

Pressure drop

Linear pressure drop is assumed.

dPdr=Pout−PinRout−Rin				(21)

(2) Parameter optimization

The basic reaction rate parameters(k0,Ei) were given in Crowe et al. Since the reactor geometry and catalyst are different from those of Crowe et al., several parameters were tuned using the plant data. In this plant, there are two reactors in series. The parameters were optimized using first 7 data (Figure 2. (a), (b)). The other data were tested using optimized parameters (Figure 2. (c), (d)).

## Simulation Results

The proposed reactor model is well fitted with the real plant data. The real data in Figures 3 and 4 show one of the plant data. Typical simulation results for the reactors are shown in Figures 3 and 4. the results are compared with the plant operating data. The performance of the model is fairly good.

Figure 3. Simulation results of first reactor

Figure 4. Simulation results of second reactor

Conclusion

The developed model shows the reactor condition well. This model can be used in optimization of SM reactor. In this reactor, the decision variables can be S/O ratio, inlet pressure of first reactor, inlet temperature of second reactor, and LHSV. The objective function is reducing the steam amount maintaining the product yield. The proposed model also can be applied to the training of reactor operators or engineers and analysis of SM reactor process.

## Acknowledgment

This work was partially supported by the Brain Korea 21 project. And thanks for the valuable technical advice provided by Dr. Jung Uk Choi in LG Chem.

## References

[1] K. M. Sundaram, H. Sardian, J. M. Fernandez-Baujin, and J. M. Hildreth, “Styrene plant simulation and optimization,” Hydrocarbon Processing, January, 93~97(1991)

[2] D. E. Clough, and W. F. Ramirez, Mathematical Modeling and Optimization of the Dehydrogenation of ethylbenzene to Form Styrene, AIChE Journal, 22, 6, 1097~1105(1976)

[3] J. G. P. Scheel, and C. M. Crowe, Simulation and optimization of an existing ethylbenzene dehydrogenation reactor, The Canadian Journal of Chemical Engineering, 47 183~187(1969)

[4] T. Hirano, Roles of potassium in potassium-promoted iron oxide catalyst for dehydrogenation of ethylbenzene, Applied Catalysis, 26, 65~79(1986)

[5] B. K. Abdalla, S. S. E. H Elnashaie, S. Alkhowaiter, S. S. Elshishini, Intrinsinc kinetics and industrial reactors modeling for the dehydrgenation of ethylbenzene to styrene on promoted iron oxide catalysts, Applied Catalysis, 113, 89~102(1994)

[6] A. A. Savoretti, D. O. Borio, V. Bucala, and J. A. Porras, Non-adiabatic radial-flow reactor for styrene production, Chemical Engineering Science, 52, 205~213(1999)