# 시스템 확인 방법에서의 흥미로운 주제

# 성수환, 이인범, 이재형*

포항공과대학교 화학공학과, 미국 Purdue 대학*

# Interesting Issues on System Identification Methods

#### Su Whan Sung, In-Beum Lee, Jay H. Lee*

Department of Chemical Engineering, Pohang University of Science and Technology,

Tel: +82-562-279-5956, FAX: +82-562-279-3499, e-mail:swsung@postech.ac.kr

School of Chemical Engineering, Purdue University, USA*

##### 1. Introduction

In this presentation, we will talk about several interesting issues on system identification. First, we will review the previous discrete-time/continuous-time system identification methods like prediction error methods, instrumental variable methods, subspace system identification method, continuous-time system identification methods using transform. Second, we will compare the continuous-time system identification methods with the discrete-time system identification methods and justifies the necessity of continuous-time system identification methods by exemplifying practical situations. Third, we will talk about the importance of choosing proper test signal for system identification. Fourth, we will present the nonlinear multivariable stochastic state space system identification as the ultimate goal of the future system identification.

2. Previous System Identification Methods

System identification methods for linear systems have been published until now can be classified into two categories of the discrete-time and continuous-time identification. The continuous-time (discrete-time) identification method is to identify the continuous-time (discrete-time) process model represented by differential (difference) equations.

2.1 Discrete-time System Identification Method

Representative discrete-time methods are prediction error methods (PEM), instrumental variable methods (IVM) and subspace system identification methods (Ljung (1987), Söderström and Stoica (1989), Forssell and Ljung (1999), Larimore (1990), Verhaegen (1994), Van Overschee and De Moor (1994)). The discrete-time (difference eqation) linear model can describe exactly the process dynamic behavior at every sample if the process input between samples is held by the zero-order holder. One example of the difference-equation model is Auto-regressive, Moving Average, eXogeneous input (ARMAX) model like the following.

A(q)y(t)=B(q)u(t)+C(q)e(t)													(1)

where, y(t),u(t) and e(t) are the process output, process input and the white noise (or prediction error), respectively.

2.1.1 Instrumental Variable Method (IVM)

The objective of IVM is estimating the deterministic part of (1), that is, A(q),B(q). To do that, we use “instrument” which is uncorrelated with the stochastic part of (1), that is, C(q)e(t) and is correlated with the regression vector to prevent singularity. Here, if the instrument is the same with the regression vector the solution of IVM becomes that of the least squares method.

2.1.2 Prediction Error Method (PEM)

The objective of PEMs is obtaining the system parameters of A(q),B(q) and C(q) minimizing the prediction error of the optimal one-step-ahead predictor. For the ARMAX model, the optimal predictor (here, “optimal” means using all possible information to predict the process output) is

y(k|k−1)=B(q)u(k)+(1−A(q))y(k)+(C(q)−1)ε(k),ε(k)=y(k)−y(k|k−1)			(2)

Then, PEM for the ARMAX model estimates the system parameters minimizing the prediction error.

MINA,B,C1Nk=1Nε2(k) s.t. (2)													(3)

To solve the nonlinear optimization problem of (3), Newton method or multi-stage least squares method can be used.

2.1.3 Subspace System Identification Method

Discrete-time subspace system identification methods have attracted much attention during the past few years due to ability of identifying multivariable linear processes directly from the input-output data. Compared with the classical PEM and IVM, these subspace methods do not suffer from a parameterization and nonlinear optimization. Also, their properties and common features have been analyzed well (Van Overshee and De Moor (1995)) and their extensions to closed-loop process data have been developed (Chou and Verhaegen (1997), Ljung and McKelvey (1996)). The objective of discrete-time subspace system identification methods is identifying the system matrices, A,B,C and K as well as the process order of the following discrete-time state space model.

x(t+1)=Ax(t)+Bu(t)+Ke(t)		(4)				y(t)=Cx(t)+e(t)			(5)

2.2 Continuous-time System Identification Method

The objective of continuous-time system identification methods is to estimate the system parameters of the process model represented by linear differential equations. For example, the continuous-time ARMAX model would be like this.

andny(t)dtn+⋯+a1dy(t)dt+y(t)=bmdmu(t)dtm+⋯+b1du(t)dt+b0u(t)+cldle(t)dtl+⋯+c1de(t)dt+e(t)

And the continuous-time state space model would be like this.

dx(t)dt=Ax(t)+Bu(t)+Ke(t)		(6) 				y(t)=Cx(t)+e(t)		(7)

Many continuous-time system identification methods have been proposed like using block pulse functions (Cheng and Hsu (1982)), multiple integrators (Whitfield and Messali (1987)). These functions produce initial value problem. So, the above-mentioned methods cannot incorporate unknown initial conditions effectively. To overcome this, linear integral transform or macro-difference expressions (Sagara and Zhao (1989), Eitelberg (1988)), digital filtering approaches (Sagara et al. (1991)) and ball-shape weighting integral transform (Sung et al. (1998)) have been developed. Sung et al. (2000) proposed continuous-time subspace system identification method.

3. Interesting Issues on System Identification

Discrete-time prediction error methods and discrete-time subspace methods have been developed so well that they look like panaceas and researchers having interests on other topics have been overwhelmed. This is not so desirable when considering potential values of other topics. We need to pay attention to topics have not attracted much attention until now. Nobody can say there are no fruitful results before exerting their efforts. In my point of view, the following several interesting issues are potential golden gooses in improving identification results.

3.1 Necessity of continuous-time system identification method

All discrete-time methods identify a discrete-time state space model. Considering most processes are controlled by digital computers with zero-order holder, the discrete-time identification approach is a good choice to identify the dynamic behavior of actual processes. However, this is not always true. We analyze the practical problems of discrete-time identification methods: If the process input is a continuous-time signal (for example, cascade control system) we cannot use discrete-time models with usually chosen sampling period. On the other hand, if we decrease the sampling period to approximate the inter-sample dynamics the identified estimates are very sensitive to uncertainties like measurement noises or structural uncertainties and round-off errors. The uncertainty bound of estimates in continuous-time system identification becomes smaller as decreasing the sampling period. Contrarily, the bound in discrete-time system identification increases as decreasing the sampling period. From simulation studies, prediction error methods as well as subspace system identification methods show totally unacceptable performances for nearly negligible uncertainties when the sampling period is small. Contrarily, continuous-time subspace identification method shows better results for smaller sampling period.

3.2 Test signal generation

In practice, there are many candidates in choosing the test signal (Cutler and Ramaker (1979), Godfrey(1993), Astrom and Hagglund (1984)). Choosing test signals can produce much bigger different identification results than we might think (Zhu et al. (1995), Kaljurand et al. (1996), Sung and Lee (1999)). For example, consider the identification of the Finite Step Response (FSR) model for Model Predictive Control (MPC). In this case, use of Pseudo-Random Binary Sequence (PRBS) can guarantee better results compared with RBS or step signal. Moreover, different design parameters of the PRBS signals result in different identification performances. Especially, for small number of samples, the differences may be really remarkable. But, we have not paid much attention to the design of the test signal generation compared with the importance.

3.3 Nonlinear Multivariable Stochastic State Space System Identification

Let us ask like this. What is the ultimate goal of the system identification? In our opinion, the answer is estimating the nonlinear multivariable stochastic state space model. We justify the statement like the following. 1. The reason of adopting a stochastic model is to include the previous prediction errors in predicting the future process output. By doing this, we can improve much more the prediction accuracy. 2. Big processes have many process outputs and process inputs and interactions between process outputs. 3. Actual processes are nonlinear. 4. For multivariable processes, state space models can represent the dynamic behavior more compactly (means using smaller adjustable parameters) compared with time-series models. Several approaches using time-series model (Nerrand et al. (1994), Chen et al. (1990)) have been proposed to identify nonlinear processes. Several authors introduced recurrent neural network models to parameterize the nonlinear state space model implicitly or explicitly (Srinivasan et al. (1994), Cheng et al. (1995), Parlos et al. (1994), Chong and Parlos (1997)). But, most of them don't use the previous prediction errors. Suykens et al. (1995) parametrized the extended Kalman filter using recurrent neural network. But, Sung et al. (2000) parameterize more general predictor form than the extended Kalman filter. Surely, we should still solve several problems like local minima, complexity, computation load etc. to achieve the goal. We hope remarkable merits of the nonlinear multivariable stochastic state space model are realized some day by solving the problem one by one.

4. References

Astrom K.J. and Hagglund T., Automatic tuning of simple regulators with specifications on phase and amplitude margins, Automatica, 1984, 20, 645

Chen S., Billings S.A. and Grant P.M., Non-linear system identification using neural networks, Int. J. Control, 1990, 51, 1191-1214

Cheng Yi, Karjala Thomas W. and David Himmelblau, Identification of nonlinear dynamic processes with unknown and variable dead time using an internal recurrent neural network, Ind. Eng. Chem. Res., 1995, 34, 1735-1742

Cheng B. and Hsu N. Analysis and parameter estimation of bilinear systems via block-pulse functions, Int. J. Control, 1982, 36, 53-65

Chong Kil To and Parlos Alexander, Comparison of the traditional and the neural networks approaches in a stochastic nonlinear system identification, Proceedings of the American Control Conference, Albuquerque, New Mexico,1997,1074-1075

Chou C.T. and Verhagegen M. Subspace algorithms for the identification of multivariable dynamic errors-in-variables models, Automatica, 1997, 33, 1857-1869

Cutler C.R. and Ramaker B.L. Dynamic matrix control-A computer control algorithm, AIChE 86th National Meeting, Houston,TX., 1979

Eitelberg E. Continuous-time system representation with exact macro-difference expressions, Int. J. Control, 1988, 47, 1207-1212

Forssell U. and Ljung L. Closed-loop identification revisited, Automatica, 1999, 35, 1215-1241

Hwang C. and Guo T. Transfer-function matrix identification in MIMO systems via shifted Godfrey K.R. Perturbation Signals for System Identification, Prentice-Hall, Englewood Cliffs, NJ, 1993

Kaljurand M., Valentin J.R. and Shao M. Feedback shift register sequences versus uniformly distributed random sequence for correlation chromatography, Analytica Chimica Acta, 1996, 324, pp. 29-36

Larimore W.E. Canonical variate analysis in identification, filtering, and adaptive control. In Proc. 29th IEEE Conf. On Decision and Control, Honolulu, Hawaii, 1990, 596-604

Ljung L. System identification: Theory for the user. Englewood Cliffs, NJ: Prentice-Hall, 1987.

Ljung L. and McKelvey T. Subspace Identification from closed-loop data, Signal Processing, 1996, 52, 209-215

Nerrand O., Roussel-Ragot P.,Urbani D., Personnaz L. and Dreyfus G., Training recurrent neural networks: why and how? an illustration in dynamic process modeling, IEEE Transctions on Neural Networks, 1994, 5, 178-184

Parlos Alexander G., Chong Kil T. and Atiya Amir F., Application of the recurrent multilayer perceptron in modeling complex process dynamics, IEEE Transactions on Neural Networks, 1994, 5, 255-266

Sagara S. ; Yang Z. and Wada K. Recursive identification algorithms for continuous systems using an adaptive procedure, Int. J. Control., 1991, 53, 391-409

Sagara S. and Zhao Z. Recursive identification of transfer function matrix in continuous systems via linear integral filter, Int. J. Control, 1989, 50, 457-477

Söderström T. and Stoica P. System identification. Englewood Cliffs, NJ: Prentice-Hall, 1989.

Srinivasan B., Prasad U.R. and Rao N.J., Back propagation through adjoints for the identification of nonlinear dynamic systems using recurrent neural models, IEEE Transactions on Neural Networks, 1994, 5, 213-228

Sung S.W., Lee I. and Lee J., New process identification method for automatic design of PID controllers, Automatica, 1998, 34, 513-520

Sung S.W. and Lee J.H., Pseudo-random binary sequence design for finite impulse response identification, AIChE annual meeting, 1999

Sung S.W., Kawk H.J., Lee S.Y. and Lee I., Continuous-time subspace system identification method, submitted to Ind. Eng. Chem. Res., 2000

Sung S.W., Pan Y. and Lee J.H., Nonlinear system identification using feedback neural network, ADCHEM, Accepted, 2000

Suykens Johan A.K., De Moor Bart L.R. and Vandewalle Joos, Nonlinear system identification using neural state space models, applicable to robust control design, Int. J. Control, 1995, 62, 129-152

Van Overschee P. and De Moor B. N4SID: Subspace algorithms for the identification of combined deterministic-stochastic systems, Automatica, 1994, 30, 75-93

Van Overschee P. and De Moor B. A unifying theorem for three subspace system identification algorithms, Automatica, 1995, 31, 1855-1864

Verhaegen M. Identification of the deterministic part of MIMO state space model given in innovations form from input-output data, Automatica, 1994, 30, 1877-1883

Whitfield A.H. and Messali N., Integral-equation approach to system identification, Int. J. Control, 1987, 45, 1431-1445

Zhu G.G., Skelton R. and Li P. q-Markov Cover identification using pseudo-random binary signals, Int. J. Control, 1995, 62, 1273-1290