# 분자체에 캡슐화된 레늄착화합물에 의한 이산화탄소의 자외선광 활성화

황진수, 박상언

한국화학연구소,분자활성공학실

# Photocatalytic Activation of CO2 under Visible Light

# by Rhenium Complex Encapsulated in Molecular Sieves

Jin-Soo Hwang and Sang-Eon Park

The photocatalytic activation of CO2 over molecular sieves-encapsulated cationic rhenium complex has been investigated under visible light (λ>350 nm). The cationic rhenium complex, [Re(I)(CO)3(bpy)(py)]+ (bpy = 2,2'-bipyridine, py = pyridine), has been encapsulated by ion-exchange method using the aqueous solution of [Re(I)(CO)3(bpy)(py)]+PF6- into the microporous NaY and the mesoporous AlMCM-41 molecular sieve acting as supramolecular heterogeneous host. To confirm the encapsulation of [Re(I)(CO)3(bpy)(py)]+ into the molecular sieves pore, Xe-NMR spectra and FT-IR spectroscopy have been measured before and after the [Re(I)(CO)3(bpy)(py)]+ encapsulation. To investigate the photophysical and photochemical properties, molecular sieves-encapsulated cationic rhenium complex has been studied by UV-Visible diffuse reflectance spectroscopy (UV-DRS) with photoirradiation (λ> 350 nm) at room temperature. By monitoring the photoreaction of CO2 over the photocatalysts, the conversion of CO2 into CO and carbonate species have been observed by using in-situ FT-IR and time-resolved mass spectroscopy. From the experimental results, the photocatalytic activation mechanism of CO2 on the catalyst under visible light (λ>350 nm) could be proposed via the photo-induced reaction of two electrons and two protons, result from water.