한정적응모델예측제어

최진훈, 고훈석, 이광순

서강대학교 화학공학과

# Constrained Adaptive Model Predictive Control

Jinhoon Choi,  Hoon Seok Ko,  Kwang Soon Lee

Department of Chemical Engineering, Sogang University

1. Introduction

Constrained model predictive control is widely accepted as a standard advanced control strategy in process industries. However, it is not an easy task to find good models for model predictive control. Moreover, the process dynamics often changes due to contamination and so on. Under these circumstances, the adaptation is quite desired in practice.

Recently, combining subspace identification and model predictive control, the so called subspace predictive control strategies are proposed [1]. In subspace identification, a optimal input-output relationship is first obtained and a state space model is found in the following steps. one can design model predictive control using the state space model from subspace identification. However, in the subspace predictive control, the optimal input-output relationship is directly used in model predictive and thus the number of steps required for model predictive control is substantially reduced. In this paper, a couple of adaptation laws are combined with this subspace predictive control and their performance are illustrated with examples.

2. Main Results

Consider the time-invariant, discrete time, linear system

xk+1=Axk+Buk+Dnk                      (1)

yk=Cxk

where xk∈Rn is the state vector; uk∈Rm is the manipulated input; yk∈Rp is the measurement output; nk∈Rq is the unknown noise. Suppose available are the input and output measurements uk,yk for k∈1,⋯,j+MN. Then the first two steps associated with subspace identification are as follows.

Step 1: Form the matrices

Up=u1u2⋯uju2u3⋯uj+1⋮⋰⋰⋮uMuM+1⋯uj+M−1,  Uf=uM+1uM+2⋯uM+juM+2uM+3⋯uM+j+1⋮⋰⋰⋮uM+NuM+N+1⋯uM+N−1.

Yp,Yf are formed similarly and Wp=YpUp.

Step 2: Find the best approximation of input output model Yf=LwWp+LuUp by solving the least square problem:

minLw,LuYf−(Lw,Lu)WpUpF2.

After these two steps, a state space model is obtained through singular value decomposition. In the subspace predictive control, the linear input output model obtained in the second step of the subspace identification is directly used for control input calculation of the model predictive control. Namely the control is computed by solving the following quadratic programming problem:

minukJ=k=1Nyk−rkTQkyk−rk+ukTRkuk

subject to input output model and input saturation constraints, where rk is the reference trajectory. In this paper we propose an adaptive version of subspace predictive control by updating the process model during the operation of the control system. Three different updating laws are considered. In the first updating law, the current process input and output data are augmented in the data matrix in the first step of subspace identification.  In the second, the current process input and output data are augmented and the oldest ones are discarded. The second technique is clearly moving window approach. In the third, the accuracy of input output model is checked every time and the current process input and output data are augmented whenever the current model is not accurate for current input output data.

3. Example

Consider the process with

A=0.92340.2310−0.5400−0.7765,B=2.32−0.897−1.4543.678,C=1111.The weighting matricies in the cost function are

Qk=1001,Rk=100010.

The input constraints we consider are −0.03≤uk≤0.03. In the simulation we have assume the system changes at k=80 to an unstable one with

A=2.2830.98−3.8−1.99,B=2.32−0.897−1.4543.678,C=1111.

Simulation results are given below.

Constrained MPC without adaptation

Constrained adaptive MPC with augmentation only

Constrained adaptive MPC with moving window

Constrained adaptive MPC with accuracy check

## Acknowledgement

Financial support from the Korean Ministry of Education through BK21 program is gratefully acknowledged.

References

[1] Favorel, W., de Moor, B. and Gevers, M. : "SPC: Subspace Predictive Control", Proceedings of 1999 IFAC World Congress, Vol. H, pp. 235-240 (1999)